CPE301 – SPRING 2019

# Design Assignment 4A

Student Name: Mohamad Jundi

Student #: 8000321867

Student Email: jundi@unlv.nevada.edu

Primary Github address: https://github.com/MohamedJundi1994/Submission_DA.git

Directory: Documents\School\CPE 301\Repository\CPE_301\DesignAssignments\DA4A

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

USB port => Xplained Mini => PD6  => TB6612FNG (PWMA)

=> TB6612FNG (VCC) => 5V source using lab board

=> TB6612FNG (VMOT) => 5V source using lab board

=> TB6612FNG (GND) => GND of Xplained and board

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

My Code:

#define F_CPU 16000000UL        // Frequency of the board

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

int adc_value;     // Used to hold ADC value

int p_button = 0;       // Used to hold button value to control motor ON/OFF

int main()

{

sei();         // Enable global interrupt

DDRC = (1 << PC2);   // Used to set PC2 as output

DDRD = (1 << PD6);   // Used to set PD6 as output

PORTC |= (1 << PC1);  // Used to enabled the pull up resistor

TCCR0A= (1 << COM0A1) | (1 << WGM01) | (1 << WGM00);  // Used to set mode to Fast PWM with clear OCR0A on match

TCCR0B= (1 << CS02) | (1 << CS00);  // Used to set the prescaler to 1024 for calculations

PCICR = (1 << PCIE1);  // PCIE1 is set high, to enable PCMSK1

PCMSK1 = (1 << PCINT9);  // PIN change interrupt source 9, so PC1 will serve as external interrupt source

ADMUX = (0<<REFS1);   // Selecting voltage reference for ADC

ADCSRA = 0b11100111;  // ADC enabled, ADC will start conversion, ADC auto trigger enabled, division factor set to 128

while (1)

{

// Infinite loop, wait

}

}

ISR(PCINT1_vect)

{

if(!(PINC & ( 1 << PC1))) // If PC1 button pressed, enter if statement

{

if(p_button == 1)  // If p_button is high, enter if statement

{

while((ADCSRA & (1 << ADIF)) == 0);  // ADC conversion completes when high

adc_value = ADC;      // The value in ADC placed in variable

OCR0A = adc_value;      // Motor will run as value of ADC set to OCR0A

_delay_ms(500);       // Used to remove denouncing

}

else if (p_button == 0) // If p_button is 0, enter if statement

{

OCR0A = 0;   // Motor will stop now

_delay_ms(500);  // Used to remove denouncing

}

p_button ^= 1;   // Reset the value of button to 1

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

All code is in number 2.

- SCHEMATICS (Battery for 5V)

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Link: https://www.youtube.com/watch?v=gCwPaDZYeIY&feature=share&fbclid=IwAR0UyIyDhQT_iGsoGexAN0O3lWAILrxfuHiE00n1zO74kOl8nfT3beN-O8A

- GITHUB LINK OF THIS DA

Link: https://github.com/MohamedJundi1994/Submission_DA.git

This assignment submission is my own, original work.

MOHAMAD JUNDI